| The engine | 103.982 |
| --- | --- |
| The amount / number of cylinders | 2960 / 6 |
| compression / compression, bar | 9.2 / >8.5 |
| Pressure oil idle [at high speed], bar | 0.3 [3.0] |
| ° oil ° C | - |
| Trap in the inlet valves mm | 0: Hyd. |
| in the final valves mm | 0: Hyd |
| Modus Operandi cylinders | 1-5-3-6-2-4 |
| Location 1st cylinder | F |
| Cooling system |  |
| The opening valve thermostat, ° C | 87±2 |
| Pressure valve corks sink, bar | 1.2±0.1 |
| Fueling |  |
| Engine idling rpm | 600-700 |
| Faster turnover idling rpm | - |
| CO content in the exhaust gas, with the engine idling [in the 3000 rpm]% | 1.0±0.5 [0.1-0.8] |
| The flow of the exhaust gases, with the engine idling [in the 3000 rpm]% | <1200 |
| The content of CO2 in the exhaust gas, with the engine idling, [with 3000 rpm]% | - |
| The O 2 in the exhaust gas, with the engine idling, [with 3000 rpm]% | - |
| Carburetor [the injection] | Bosch |
| Type of device | CIS-E (KE-Jetronic) |
| Chief jikler / dispenser | - |
| The injection pressure, bar | >3.0> 3.0 |
| Pressure fuel bar | 5.3-5.55.3 |
| Octanenumber petrol, RON | 95[E 95 RON] |
| Ignition system |  |
| Type | EZL |
| Reel | Bosch |
| Resistance primary windings, Om | 0.3-0.6 |
| Complementary resistance Om | - |
| Voltage <kontakt 15(+) - zemla> | Battery |
| Switchboard | Bosch |
| Point gap [air gap] [mm] | - |
| Angle circuit breaker contacts, ° (%) | 1-30 @ starter speed |
| Capacitance of the unit, F | - |
| Direction of rotation window | - |
| Moment ignition-start (° to door-rpm) | 27-31 BTDC @ 3200 |
| The vacuum controller | NV |
| The total ignition timing advance, (° to door-rpm) | 6-11 BTDC @ idle |
|  | 40-44 BTDC @ 3200 |
|  | - |
| Data for the centrifugal regulator (° to door-rpm) | Computer control |
|  | - |
|  | - |
| Data for the vacuum regulator, Mbar | Computer control |
| Maximum ignition timing advance, ° | - |
| Mark sparkplugs | Bosch / Champion |
| Type | H7DC / S9YC |
| Gap between electrodes mm | 0.80.8 |