1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

ADS unit

car front

ADS control unit pin assignment

1:	Power supply, 12V (overvoltage protection relay)

2:	Power supply 12V (from alternator)

3:	-

4:	to Diagnostic terminal (pin #9, 12V)

5:	Left front wheel speed sensor (alternating current)

6:	Oil level switch 12V

7:	ADS warning lamp (dashboard, 12V when light is off)

8:	Wheel acceleration pickup (power supply?)

9:	Body acceleration pickup (power supply?)

10:	Level adjustment control valve, lower

11:	Level adjustment control valve, raise

12:	Rear axle solenoid valve 2

13:	-

14:	Rear axle solenoid valve 2

15:	-

16:	Front axle solenoid valve 2 right

17:	Front axle solenoid valve 2 left

18:	Front axle solenoid valve 2 right

19:	Front axle solenoid valve 2 left

20:	-

21:	Steering angle sensor (ac)

22:	Hard/soft switch

23:	Ground

24:	Wheel accelereation pickup

25:	Wheel accelereation pickup (sensor signal off > dc –

sensor signal dynamic > ac)

26:	Body acceleration pickup (sensor signal off > dc –

sensor signal dynamic > ac)

27:	Body acceleration pickup

28:	Level switch raise

29:	Level switch lower

30:	Hard/soft switch

31:	Level adjustment control valve

32:	Rear axle solenoid valve (power supply?)

33:	-

34:	Front axle solenoid valve, right (power supply?)

35:	Front axle solenoid valve, left (power supply?)